Application：

The   cable are intended for telecommunication, telecommunication, computer and   information proceeding unit of shipboard and control system.

| 型号 Type | 名称 Description | 产品标准 Standard |
| --- | --- | --- |
| CHEF/NA | 乙丙绝缘氯丁护套船用对称式通信电缆，NA型 EPR insulation PCP sheath shipboard symmetrical communication cable ,Type NA | IEC60092-350 IEC60092-376 IEC60331 IEC60332-3A |
| CHEF80/NA | 乙丙绝缘氯丁内护套镀锡铜丝编织铠装船用对称式通信电缆，NA型 EPR insulation PCP inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type NA |  |
| CHEF90/NA | 乙丙绝缘氯丁内护套镀锌钢丝编织铠装船用对称式通信电缆，NA型 EPR insulation PCP inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type NA |  |
| CHEF82/NA | 乙丙绝缘氯丁内护套镀锡铜丝编织铠装聚氯乙烯外护套船用对称式通信电缆，NA型 EPR insulation PCP inner sheath tinned copper wire braided PVC outer sheath shipboard symmetrical communication cable ,Type NA |  |
| CHEF92/NA | 乙丙绝缘氯丁内护套镀锌钢丝编织铠装聚氯乙烯外护套船用对称式通信电缆，NA型 EPR insulation PCP inner sheath galvanized steel wire braided PVC outer sheath shipboard symmetrical communication cable ,Type NA |  |
| CHEFP/NA | 乙丙绝缘镀锡铜丝编织独立屏蔽氯丁护套船用对称式通信电缆，NA型 EPR insulation tinned copper wire braided individual screen PCP sheath shipboard symmetrical communication cable ,Type NA |  |
| CHEFP80/NA | 乙丙绝缘镀锡铜丝编织独立屏蔽氯丁内护套镀锡铜丝编织铠装船用对称式通信电缆，NA型 EPR insulation tinned copper wire braided individual screen PCP inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type NA |  |
| CHEFP90/NA | 乙丙绝缘镀锡铜丝编织独立屏蔽氯丁内护套镀锌钢丝编织铠装船用对称式通信电缆，NA型 EPR insulation tinned copper wire braided individual screen PCP inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type NA |  |
| CHEFP82/NA | 乙丙绝缘镀锡铜丝编织独立屏蔽氯丁内护套镀锡铜丝编织铠装聚氯乙烯外护套船用对称式通信电缆，NA型 EPR insulation tinned copper wire braided individual screen PCP inner sheath tinned copper wire braided PVC outer sheath shipboard symmetrical communication cable ,Type NA |  |
| CHEFP92/NA | 乙丙绝缘镀锡铜丝编织独立屏蔽氯丁内护套镀锌钢丝编织铠装聚氯乙烯外护套船用对称式通信电缆，NA型 EPR insulation tinned copper wire braided individual screen PCP inner sheath galvanized steel wire braided PVC outer sheath shipboard symmetrical communication cable ,Type NA |  |

| 型号 Type | 名称 Description | 产品标准 Standard |
| --- | --- | --- |
| CHEV/NA | 乙丙绝缘聚氯乙烯护套船用对称式通信电缆，NA型 EPR insulation PVC sheath shipboard symmetrical communication cable ,Type NA | IEC60092-350 IEC60092-376 IEC60331 IEC60332-3A |
| CHEV80/NA | 乙丙绝缘聚氯乙烯内护套镀锡铜丝编织铠装船用对称式通信电缆，NA型 EPR insulation PVC inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type NA |  |
| CHEV90/NA | 乙丙绝缘聚氯乙烯内护套镀锌钢丝编织铠装船用对称式通信电缆，NA型 EPR insulation PVC inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type NA |  |
| CHEV82/NA | 乙丙绝缘聚氯乙烯内护套镀锡铜丝编织铠装聚氯乙烯外护套船用对称式通信电缆，NA型 EPR insulation PVC inner sheath tinned copper wire braided PVC outer sheath shipboard symmetrical communication cable ,Type NA |  |
| CHEV92/NA | 乙丙绝缘聚氯乙烯内护套镀锌钢丝编织铠装聚氯乙烯外护套船用对称式通信电缆，NA型 EPR insulation PVC inner sheath galvanized steel wire braided PVC outer sheath shipboard symmetrical communication cable ,Type NA |  |
| CHEVP/NA | 乙丙绝缘镀锡铜丝编织独立屏蔽聚氯乙烯护套船用对称式通信电缆，NA型 EPR insulation tinned copper wire braided individual screen PVC sheath shipboard symmetrical communication cable ,Type NA |  |
| CHEVP80/NA | 乙丙绝缘镀锡铜丝编织独立屏蔽聚氯乙烯内护套镀锡铜丝编织铠装船用对称式通信电缆，NA型 EPR insulation tinned copper wire braided individual screen PVC inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type NA |  |
| CHEVP90/NA | 乙丙绝缘镀锡铜丝编织独立屏蔽聚氯乙烯内护套镀锌钢丝编织铠装船用对称式通信电缆，NA型 EPR insulation tinned copper wire braided individual screen PVC inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type NA |  |
| CHEVP82/NA | 乙丙绝缘镀锡铜丝编织独立屏蔽聚氯乙烯内护套镀锡铜丝编织铠装聚氯乙烯外护套船用对称式通信电缆，NA型 EPR insulation tinned copper wire braided individual screen PVC inner sheath tinned copper wire braided PVC outer sheath shipboard symmetrical communication cable ,Type NA |  |
| CHEVP92/NA | 乙丙绝缘镀锡铜丝编织独立屏蔽聚氯乙烯内护套镀锌钢丝编织铠装聚氯乙烯外护套船用对称式通信电缆，NA型 EPR insulation tinned copper wire braided individual screen PVC inner sheath galvanized steel wire braided PVC outer sheath shipboard symmetrical communication cable ,Type NA |  |

| 型号 Type | 名称 Description | 产品标准 Standard |
| --- | --- | --- |
| CHVV/NA | 聚氯乙烯绝缘聚氯乙烯护套船用对称式通信电缆，NA型 PVC insulation PVC sheath shipboard symmetrical communication cable ,Type NA | IEC60092-350 IEC60092-376 IEC60331 IEC60332-3A |
| CHVV80/NA | 聚氯乙烯绝缘聚氯乙烯内护套镀锡铜丝编织铠装船用对称式通信电缆，NA型 PVC insulation PVC inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type NA |  |
| CHVV90/NA | 聚氯乙烯绝缘聚氯乙烯内护套镀锌钢丝编织铠装船用对称式通信电缆，NA型 PVC insulation PVC inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type NA |  |
| CHVV82/NA | 聚氯乙烯绝缘聚氯乙烯内护套镀锡铜丝编织铠装聚氯乙烯外护套船用对称式通信电缆，NA型 PVC insulation PVC inner sheath tinned copper wire braided PVC outer sheath shipboard symmetrical communication cable ,Type NA |  |
| CHVV92/NA | 聚氯乙烯绝缘聚氯乙烯内护套镀锌钢丝编织铠装聚氯乙烯外护套船用对称式通信电缆，NA型 PVC insulation PVC inner sheath galvanized steel wire braided PVC outer sheath shipboard symmetrical communication cable ,Type NA |  |
| CHVVP/NA | 聚氯乙烯绝缘镀锡铜丝编织独立屏蔽聚氯乙烯护套船用对称式通信电缆，NA型 PVC insulation tinned copper wire braided individual screen PVC sheath shipboard symmetrical communication cable ,Type NA |  |
| CHVVP80/NA | 聚氯乙烯绝缘镀锡铜丝编织独立屏蔽聚氯乙烯内护套镀锡铜丝编织铠装船用对称式通信电缆，NA型 PVC insulation tinned copper wire braided individual screen PVC inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type NA |  |
| CHVVP90/NA | 聚氯乙烯绝缘镀锡铜丝编织独立屏蔽聚氯乙烯内护套镀锌钢丝编织铠装船用对称式通信电缆，NA型 PVC insulation tinned copper wire braided individual screen PVC inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type NA |  |
| CHVVP82/NA | 聚氯乙烯绝缘镀锡铜丝编织独立屏蔽聚氯乙烯内护套镀锡铜丝编织铠装聚氯乙烯外护套船用对称式通信电缆，NA型 PVC insulation tinned copper wire braided individual screen PVC inner sheath tinned copper wire braided PVC outer sheath shipboard symmetrical communication cable ,Type NA |  |
| CHVVP92/NA | 聚氯乙烯绝缘镀锡铜丝编织独立屏蔽聚氯乙烯内护套镀锌钢丝编织铠装聚氯乙烯外护套船用对称式通信电缆，NA型 PVC insulation tinned copper wire braided individual screen PVC inner sheath galvanized steel wire braided PVC outer sheath shipboard symmetrical communication cable ,Type NA |  |
| CHV8290/NA | 聚氯乙烯绝缘镀锡铜丝编织铠装聚氯乙烯内护套镀锌钢丝编织铠装船用对称式通信电缆，NA型 PVC insulation tinned copper wire braided PVC inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type NA |  |
| CHV8292/NA | 聚氯乙烯绝缘镀锡铜丝编织铠装聚氯乙烯内护套镀锌钢丝编织铠装聚氯乙烯外护套船用对称式通信电缆，NA型 PVC insulation tinned copper wire braided PVC inner sheath galvanized steel wire braided PVC outer sheath shipboard symmetrical communication cable ,Type NA |  |

| 型号 Type | 名称 Description | 产品标准 Standard |
| --- | --- | --- |
| CHEPJ/NC | 乙丙绝缘交联聚烯烃护套船用对称式通信电缆，NC型 EPR insulation cross-linked PO sheath shipboard symmetrical communication cable ,Type NC | IEC60092-350 IEC60092-376 IEC60331 IEC60332-3A |
| CHEPJ80/NC | 乙丙绝缘交联聚烯烃内护套镀锡铜丝编织铠装船用对称式通信电缆，NC型 EPR insulation cross-linked PO inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type NC |  |
| CHEPJ90/NC | 乙丙绝缘交联聚烯烃内护套镀锌钢丝编织铠装船用对称式通信电缆，NC型 EPR insulation cross-linked PO inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type NC |  |
| CHEPJ85/NC | 乙丙绝缘交联聚烯烃内护套镀锡铜丝编织铠装交联聚烯烃外护套船用对称式通信电缆，NC型 EPR insulation cross-linked PO inner sheath tinned copper wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type NC |  |
| CHEPJ95/NC | 乙丙绝缘交联聚烯烃内护套镀锌钢丝编织铠装交联聚烯烃外护套船用对称式通信电缆，NC型 EPR insulation cross-linked PO inner sheath galvanized steel wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type NC |  |
| CHEPJP/NC | 乙丙绝缘镀锡铜丝编织独立屏蔽交联聚烯烃护套船用对称式通信电缆，NC型 EPR insulation tinned copper wire braided individual screen cross-linked PO sheath shipboard symmetrical communication cable ,Type NC |  |
| CHEPJP80/NC | 乙丙绝缘镀锡铜丝编织独立屏蔽交联聚烯烃内护套镀锡铜丝编织铠装船用对称式通信电缆，NC型 EPR insulation tinned copper wire braided individual screen cross-linked PO inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type NC |  |
| CHEPJP90/NC | 乙丙绝缘镀锡铜丝编织独立屏蔽交联聚烯烃内护套镀锌钢丝编织铠装船用对称式通信电缆，NC型 EPR insulation tinned copper wire braided individual screen cross-linked PO inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type NC |  |
| CHEPJP85/NC | 乙丙绝缘镀锡铜丝编织独立屏蔽交联聚烯烃内护套镀锡铜丝编织铠装交联聚烯烃外护套船用对称式通信电缆，NC型 EPR insulation tinned copper wire braided individual screen cross-linked PO inner sheath tinned copper wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type NC |  |
| CHEPJP95/NC | 乙丙绝缘镀锡铜丝编织独立屏蔽交联聚烯烃内护套镀锌钢丝编织铠装交联聚烯烃外护套船用对称式通信电缆，NC型 EPR insulation tinned copper wire braided individual screen cross-linked PO inner sheath galvanized steel wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type NC |  |

| 型号 Type | 名称 Description | 产品标准 Standard |
| --- | --- | --- |
| CHEPF/NC | 乙丙绝缘聚烯烃护套船用对称式通信电缆，NC型 EPR insulation PO sheath shipboard symmetrical communication cable ,Type NC | IEC60092-350 IEC60092-376 IEC60331 IEC60332-3A |
| CHEPF80/NC | 乙丙绝缘聚烯烃内护套镀锡铜丝编织铠装船用对称式通信电缆，NC型 EPR insulation PO inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type NC |  |
| CHEPF90/NC | 乙丙绝缘聚烯烃内护套镀锌钢丝编织铠装船用对称式通信电缆，NC型 EPR insulation PO inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type NC |  |
| CHEPF86/NC | 乙丙绝缘聚烯烃内护套镀锡铜丝编织铠装交联聚烯烃外护套船用对称式通信电缆，NC型 EPR insulation PO inner sheath tinned copper wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type NC |  |
| CHEPF96/NC | 乙丙绝缘聚烯烃内护套镀锌钢丝编织铠装交联聚烯烃外护套船用对称式通信电缆，NC型 EPR insulation PO inner sheath galvanized steel wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type NC |  |
| CHEPFP/NC | 乙丙绝缘镀锡铜丝编织独立屏蔽聚烯烃护套船用对称式通信电缆，NC型 EPR insulation tinned copper wire braided individual screen PO sheath shipboard symmetrical communication cable ,Type NC |  |
| CHEPFP80/NC | 乙丙绝缘镀锡铜丝编织独立屏蔽聚烯烃内护套镀锡铜丝编织铠装船用对称式通信电缆，NC型 EPR insulation tinned copper wire braided individual screen PO inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type NC |  |
| CHEPFP90/NC | 乙丙绝缘镀锡铜丝编织独立屏蔽聚烯烃内护套镀锌钢丝编织铠装船用对称式通信电缆，NC型 EPR insulation tinned copper wire braided individual screen PO inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type NC |  |
| CHEPFP86/NC | 乙丙绝缘镀锡铜丝编织独立屏蔽聚烯烃内护套镀锡铜丝编织铠装聚烯烃外护套船用对称式通信电缆，NC型 EPR insulation tinned copper wire braided individual screen PO inner sheath tinned copper wire braided PO outer sheath shipboard symmetrical communication cable ,Type NC |  |
| CHEPFP96/NC | 乙丙绝缘镀锡铜丝编织独立屏蔽聚烯烃内护套镀锌钢丝编织铠装聚烯烃外护套船用对称式通信电缆，NC型 EPR insulation tinned copper wire braided individual screen PO inner sheath galvanized steel wire braided PO outer sheath shipboard symmetrical communication cable ,Type NC |  |

| 型号 Type | 名称 Description |
| --- | --- |
| CHJV/NA | 交联聚乙烯绝缘聚氯乙烯护套船用对称式通信电缆，NA型 XLPE insulation PVC sheath shipboard symmetrical communication cable ,Type NA |
| CHJV80/NA | 交联聚乙烯绝缘聚氯乙烯内护套镀锡铜丝编织铠装船用对称式通信电缆，NA型 XLPE insulation PVC inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type NA |
| CHJV90/NA | 交联聚乙烯绝缘聚氯乙烯内护套镀锌钢丝编织铠装船用对称式通信电缆，NA型 XLPE insulation PVC inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type NA |
| CHJV82/NA | 交联聚乙烯绝缘聚氯乙烯内护套镀锡铜丝编织铠装聚氯乙烯外护套船用对称式通信电缆，NA型 XLPE insulation PVC inner sheath tinned copper wire braided PVC outer sheath shipboard symmetrical communication cable ,Type NA |
| CHJV92/NA | 交联聚乙烯绝缘聚氯乙烯内护套镀锌钢丝编织铠装聚氯乙烯外护套船用对称式通信电缆，NA型 XLPE insulation PVC inner sheath galvanized steel wire braided PVC outer sheath shipboard symmetrical communication cable ,Type NA |
| CHJVP/NA | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚氯乙烯护套船用对称式通信电缆，NA型 XLPE insulation tinned copper wire braided individual screen PVC sheath shipboard symmetrical communication cable ,Type NA |
| CHJVP80/NA | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚氯乙烯内护套镀锡铜丝编织铠装船用对称式通信电缆，NA型 XLPE insulation tinned copper wire braided individual screen PVC inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type NA |
| CHJVP90/NA | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚氯乙烯内护套镀锌钢丝编织铠装船用对称式通信电缆，NA型 XLPE insulation tinned copper wire braided individual screen PVC inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type NA |
| CHJVP82/NA | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚氯乙烯内护套镀锡铜丝编织铠装聚氯乙烯外护套船用对称式通信电缆，NA型 XLPE insulation tinned copper wire braided individual screen PVC inner sheath tinned copper wire braided PVC outer sheath shipboard symmetrical communication cable ,Type NA |
| CHJVP92/NA | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚氯乙烯内护套镀锌钢丝编织铠装聚氯乙烯外护套船用对称式通信电缆，NA型 XLPE insulation tinned copper wire braided individual screen PVC inner sheath galvanized steel wire braided PVC outer sheath shipboard symmetrical communication cable ,Type NA |

| 型号 Type | 名称 Description | 产品标准 Standard |
| --- | --- | --- |
| CHJPJ/NC | 交联聚乙烯绝缘交联聚烯烃护套船用对称式通信电缆，NC型 XLPE insulation cross-linked PO sheath shipboard symmetrical communication cable ,Type NC | IEC60092-350 IEC60092-376 IEC60331 IEC60332-3A |
| CHJPJ80/NC | 交联聚乙烯绝缘交联聚烯烃内护套镀锡铜丝编织铠装船用对称式通信电缆，NC型 XLPE insulation cross-linked PO inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type NC |  |
| CHJPJ90/NC | 交联聚乙烯绝缘交联聚烯烃内护套镀锌钢丝编织铠装船用对称式通信电缆，NC型 XLPE insulation cross-linked PO inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type NC |  |
| CHJPJ85/NC | 交联聚乙烯绝缘交联聚烯烃内护套镀锡铜丝编织铠装交联聚烯烃外护套船用对称式通信电缆，NC型 EPR insulation cross-linked PO inner sheath tinned copper wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type NC |  |
| CHJPJ95/NC | 交联聚乙烯绝缘交联聚烯烃内护套镀锌钢丝编织铠装交联聚烯烃外护套船用对称式通信电缆，NC型 XLPE insulation cross-linked PO inner sheath galvanized steel wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type NC |  |
| CHJPJP/NC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽交联聚烯烃护套船用对称式通信电缆，NC型 EPR insulation tinned copper wire braided individual screen cross-linked PO sheath shipboard symmetrical communication cable ,Type NC |  |
| CHJPJP80/NC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽交联聚烯烃内护套镀锡铜丝编织铠装船用对称式通信电缆，NC型 XLPE insulation tinned copper wire braided individual screen cross-linked PO inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type NC |  |
| CHJPJP90/NC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽交联聚烯烃内护套镀锌钢丝编织铠装船用对称式通信电缆，NC型 XLPE insulation tinned copper wire braided individual screen cross-linked PO inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type NC |  |
| CHJPJP85/NC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽交联聚烯烃内护套镀锡铜丝编织铠装交联聚烯烃外护套船用对称式通信电缆，NC型 XLPE insulation tinned copper wire braided individual screen cross-linked PO inner sheath tinned copper wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type NC |  |
| CHJPJP95/NC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽交联聚烯烃内护套镀锌钢丝编织铠装交联聚烯烃外护套船用对称式通信电缆，NC型 XLPE insulation tinned copper wire braided individual screen cross-linked PO inner sheath galvanized steel wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type NC |  |
| CHJ85/NC | 交联聚乙烯绝缘镀锡铜丝编织铠装交联聚烯烃外护套船用对称式通信电缆，NC型 EPR insulation tinned copper wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type NC |  |
| CHJP85/NC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽镀锡铜丝编织铠装交联聚烯烃外护套船用对称式通信电缆，NC型 XLPE insulation tinned copper wire braided individual screen tinned copper wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type NC |  |

| 型号 Type | 名称 Description | 产品标准 Standard |
| --- | --- | --- |
| CHJPF/NC | 交联聚乙烯绝缘聚烯烃护套船用对称式通信电缆，NC型 XLPE insulation PO sheath shipboard symmetrical communication cable ,Type NC | IEC60092-350 IEC60092-376 IEC60331 IEC60332-3A |
| CHJPF80/NC | 交联聚乙烯绝缘聚烯烃内护套镀锡铜丝编织铠装船用对称式通信电缆，NC型 XLPE insulation PO inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type NC |  |
| CHJPF90/NC | 交联聚乙烯绝缘聚烯烃内护套镀锌钢丝编织铠装船用对称式通信电缆，NC型 XLPE insulation PO inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type NC |  |
| CHJPF86/NC | 交联聚乙烯绝缘聚烯烃内护套镀锡铜丝编织铠装聚烯烃外护套船用对称式通信电缆，NC型 XLPE insulation PO inner sheath tinned copper wire braided PO outer sheath shipboard symmetrical communication cable ,Type NC |  |
| CHJPF96/NC | 交联聚乙烯绝缘聚烯烃内护套镀锌钢丝编织铠装聚烯烃外护套船用对称式通信电缆，NC型 XLPE insulation PO inner sheath galvanized steel wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type NC |  |
| CHJPFP/NC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚烯烃护套船用对称式通信电缆，NC型 XLPE insulation tinned copper wire braided individual screen PO sheath shipboard symmetrical communication cable ,Type NC |  |
| CHJPFP80/NC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚烯烃内护套镀锡铜丝编织铠装船用对称式通信电缆，NC型 XLPE insulation tinned copper wire braided individual screen PO inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type NC |  |
| CHJPFP90/NC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚烯烃内护套镀锌钢丝编织铠装船用对称式通信电缆，NC型 XLPE insulation tinned copper wire braided individual screen PO inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type NC |  |
| CHJPFP86/NC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚烯烃内护套镀锡铜丝编织铠装聚烯烃外护套船用对称式通信电缆，NC型 XLPE insulation tinned copper wire braided individual screen PO inner sheath tinned copper wire braided PO outer sheath shipboard symmetrical communication cable ,Type NC |  |
| CHJPFP96/NC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚烯烃内护套镀锌钢丝编织铠装聚烯烃外护套船用对称式通信电缆，NC型 XLPE insulation tinned copper wire braided individual screen PO inner sheath galvanized steel wire braided PO outer sheath shipboard symmetrical communication cable ,Type NC |  |
| CHJ86/NC | 交联聚乙烯绝缘镀锡铜丝编织铠装聚烯烃外护套船用对称式通信电缆，NC型 XLPE insulation PO inner sheath tinned copper wire braided PO outer sheath shipboard symmetrical communication cable ,Type NC |  |
| CHJP86/NC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽镀锡铜丝编织铠装聚烯烃外护套船用对称式通信电缆，NC型 XLPE insulation tinned copper wire braided individual screen PO inner sheath tinned copper wire braided PO outer sheath shipboard symmetrical communication cable ,Type NC |  |

| 型号 Type | 名称 Description | 产品标准 Standard |
| --- | --- | --- |
| CHJP85/NC （铝塑复合箔独立屏蔽） | 交联聚乙烯绝缘铝塑复合箔加泄漏线独立屏蔽镀镀锡铜丝编织铠装交联聚烯烃外护套船用对称式通信电缆，NC型 XLPE insulation aluminium/polyester foil with tinned copper drain wire individual screen tinned copper wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type NC | IEC60092-350 IEC60092-376 IEC60331 IEC60332-3A |
| CHJP86/NC（铝塑复合箔独立屏蔽） | 交联聚乙烯绝缘铝塑复合箔加泄漏线独立屏蔽镀锡铜丝编织铠装聚烯烃外护套船用对称式通信电缆，NC型 XLPE insulation aluminium/polyester foil with tinned copper drain wire individual screen tinned copper wire braided PO outer sheath shipboard symmetrical communication cable ,Type NC |  |
| CHJPJP85/NC（铝塑复合箔独立屏蔽） | 交联聚乙烯绝缘铝塑复合箔加泄漏线独立屏蔽交联聚烯烃内护套镀锡铜丝编织铠装交联聚烯烃外护套船用对称式通信电缆，NC型 XLPE insulation aluminium/polyester foil with tinned copper drain wire individual screen cross-linked PO inner sheath tinned copper wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type NC |  |
| CHJPJP85/NC（铝塑复合箔整体屏蔽） | 交联聚乙烯绝缘铝塑复合箔加泄漏线整体屏蔽交联聚烯烃内护套镀锡铜丝编织铠装交联聚烯烃外护套船用对称式通信电缆，NC型 XLPE insulation aluminium/polyester foil with tinned copper drain wire collectively screen cross-linked PO inner sheath tinned copper wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type NC |  |
| CHJPJP85/NC（铝塑复合箔独立+整体屏蔽） | 交联聚乙烯绝缘铝塑复合箔加泄漏线独立屏蔽整体屏蔽交联聚烯烃内护套镀锡铜丝编织铠装交联聚烯烃外护套船用对称式通信电缆，NC型 XLPE insulation aluminium/polyester foil with tinned copper drain wire individual and collectively screen cross-linked PO inner sheath tinned copper wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type NC |  |
| CHJPFP86/NC（铝塑复合箔独立屏蔽） | 交联聚乙烯绝缘铝塑复合箔加泄漏线独立屏蔽聚烯烃内护套镀锡铜丝编织铠装聚烯烃外护套船用对称式通信电缆，NC型 XLPE insulation aluminium/polyester foil with tinned copper drain wire individual screen PO inner sheath tinned copper wire braided PO outer sheath shipboard symmetrical communication cable ,Type NC |  |
| CHJPFP86/NC（铝塑复合箔整体屏蔽） | 交联聚乙烯绝缘铝塑复合箔加泄漏线整体屏蔽聚烯烃内护套镀锡铜丝编织铠装聚烯烃外护套船用对称式通信电缆，NC型 XLPE insulation aluminium/polyester foil with tinned copper drain wire collectively screen PO inner sheath tinned copper wire braided PO outer sheath shipboard symmetrical communication cable ,Type NC |  |
| CHJPFP86/NC（铝塑复合箔独立+整体屏蔽） | 交联聚乙烯绝缘铝塑复合箔加泄漏线独立屏蔽整体屏蔽聚烯烃内护套镀锡铜丝编织铠装聚烯烃外护套船用对称式通信电缆，NC型 XLPE insulation aluminium/polyester foil with tinned copper drain wire individual and collectively screen PO inner sheath tinned copper wire braided PO outer sheath shipboard symmetrical communication cable ,Type NC |  |