Standards：

IEC60092-350

IEC60092-351

IEC60092-352

IEC60332-359

IEC60332-376

IEC30228

IEC60332

Application：

These cable are intended for telecommunication, computer and information proceeding unit of shipboard and naval ship, and it is also   available to metallurgical industry chemical works, power plant and mine etc. control system or shipboard and off-shore building

1 —­­­conductor Tinned copper wire

2 —­­­Insulation EPR/PVC/XLPE

3 —Separated screen tinned copper wire braided

4 —­­­Tape

5 —­­­Filler

6 —Inner   sheath PCP//PVC

7 —­­­Armour Tinned copper wire or galvanized steel wire braid

8—­­­Outer   sheath PVC/PCP

| 型号 Type | 名称 Description | 产品标准 Standard |
| --- | --- | --- |
| CHEF/SA | 乙丙绝缘氯丁护套船用对称式通信电缆，SA型 EPR insulation PCP sheath shipboard symmetrical communication cable ,Type SA | IEC60092-350 IEC60092-376 IEC60332-3A |
| CHEF80/SA | 乙丙绝缘氯丁内护套镀锡铜丝编织铠装船用对称式通信电缆，SA型 EPR insulation PCP inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type SA |  |
| CHEF90/SA | 乙丙绝缘氯丁内护套镀锌钢丝编织铠装船用对称式通信电缆，SA型 EPR insulation PCP inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type SA |  |
| CHEF82/SA | 乙丙绝缘氯丁内护套镀锡铜丝编织铠装聚氯乙烯外护套船用对称式通信电缆，SA型 EPR insulation PCP inner sheath tinned copper wire braided PVC outer sheath shipboard symmetrical communication cable ,Type SA |  |
| CHEF92/SA | 乙丙绝缘氯丁内护套镀锌钢丝编织铠装聚氯乙烯外护套船用对称式通信电缆，SA型 EPR insulation PCP inner sheath galvanized steel wire braided PVC outer sheath shipboard symmetrical communication cable ,Type SA |  |
| CHEFP/SA | 乙丙绝缘镀锡铜丝编织独立屏蔽氯丁护套船用对称式通信电缆，SA型 EPR insulation tinned copper wire braided individual screen PCP sheath shipboard symmetrical communication cable ,Type SA |  |
| CHEFP80/SA | 乙丙绝缘镀锡铜丝编织独立屏蔽氯丁内护套镀锡铜丝编织铠装船用对称式通信电缆，SA型 EPR insulation tinned copper wire braided individual screen PCP inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type SA |  |
| CHEFP90/SA | 乙丙绝缘镀锡铜丝编织独立屏蔽氯丁内护套镀锌钢丝编织铠装船用对称式通信电缆，SA型 EPR insulation tinned copper wire braided individual screen PCP inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type SA |  |
| CHEFP82/SA | 乙丙绝缘镀锡铜丝编织独立屏蔽氯丁内护套镀锡铜丝编织铠装聚氯乙烯外护套船用对称式通信电缆，SA型 EPR insulation tinned copper wire braided individual screen PCP inner sheath tinned copper wire braided PVC outer sheath shipboard symmetrical communication cable ,Type SA |  |
| CHEFP92/SA | 乙丙绝缘镀锡铜丝编织独立屏蔽氯丁内护套镀锌钢丝编织铠装聚氯乙烯外护套船用对称式通信电缆，SA型 EPR insulation tinned copper wire braided individual screen PCP inner sheath galvanized steel wire braided PVC outer sheath shipboard symmetrical communication cable ,Type SA |  |

| 型号 Type | 名称 Description | 产品标准 Standard |
| --- | --- | --- |
| CHEV/SA | 乙丙绝缘聚氯乙烯护套船用对称式通信电缆，SA型 EPR insulation PVC sheath shipboard symmetrical communication cable ,Type SA | IEC60092-350 IEC60092-376 IEC60332-3A |
| CHEV80/SA | 乙丙绝缘聚氯乙烯内护套镀锡铜丝编织铠装船用对称式通信电缆，SA型 EPR insulation PVC inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type SA |  |
| CHEV90/SA | 乙丙绝缘聚氯乙烯内护套镀锌钢丝编织铠装船用对称式通信电缆，SA型 EPR insulation PVC inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type SA |  |
| CHEV82/SA | 乙丙绝缘聚氯乙烯内护套镀锡铜丝编织铠装聚氯乙烯外护套船用对称式通信电缆，SA型 EPR insulation PVC inner sheath tinned copper wire braided PVC outer sheath shipboard symmetrical communication cable ,Type SA |  |
| CHEV92/SA | 乙丙绝缘聚氯乙烯内护套镀锌钢丝编织铠装聚氯乙烯外护套船用对称式通信电缆，SA型 EPR insulation PVC inner sheath galvanized steel wire braided PVC outer sheath shipboard symmetrical communication cable ,Type SA |  |
| CHEVP/SA | 乙丙绝缘镀锡铜丝编织独立屏蔽聚氯乙烯护套船用对称式通信电缆，SA型 EPR insulation tinned copper wire braided individual screen PVC sheath shipboard symmetrical communication cable ,Type SA |  |
| CHEVP80/SA | 乙丙绝缘镀锡铜丝编织独立屏蔽聚氯乙烯内护套镀锡铜丝编织铠装船用对称式通信电缆，SA型 EPR insulation tinned copper wire braided individual screen PVC inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type SA |  |
| CHEVP90/SA | 乙丙绝缘镀锡铜丝编织独立屏蔽聚氯乙烯内护套镀锌钢丝编织铠装船用对称式通信电缆，SA型 EPR insulation tinned copper wire braided individual screen PVC inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type SA |  |
| CHEVP82/SA | 乙丙绝缘镀锡铜丝编织独立屏蔽聚氯乙烯内护套镀锡铜丝编织铠装聚氯乙烯外护套船用对称式通信电缆，SA型 EPR insulation tinned copper wire braided individual screen PVC inner sheath tinned copper wire braided PVC outer sheath shipboard symmetrical communication cable ,Type SA |  |
| CHEVP92/SA | 乙丙绝缘镀锡铜丝编织独立屏蔽聚氯乙烯内护套镀锌钢丝编织铠装聚氯乙烯外护套船用对称式通信电缆，SA型 EPR insulation tinned copper wire braided individual screen PVC inner sheath galvanized steel wire braided PVC outer sheath shipboard symmetrical communication cable ,Type SA |  |

| 型号 Type | 名称 Description | 产品标准 Standard |
| --- | --- | --- |
| CHVV/SA | 聚氯乙烯绝缘聚氯乙烯护套船用对称式通信电缆，SA型 PVC insulation PVC sheath shipboard symmetrical communication cable ,Type SA | IEC60092-350 IEC60092-376 IEC60332-3A |
| CHVV80/SA | 聚氯乙烯绝缘聚氯乙烯内护套镀锡铜丝编织铠装船用对称式通信电缆，SA型 PVC insulation PVC inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type SA |  |
| CHVV90/SA | 聚氯乙烯绝缘聚氯乙烯内护套镀锌钢丝编织铠装船用对称式通信电缆，SA型 PVC insulation PVC inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type SA |  |
| CHVV82/SA | 聚氯乙烯绝缘聚氯乙烯内护套镀锡铜丝编织铠装聚氯乙烯外护套船用对称式通信电缆，SA型 PVC insulation PVC inner sheath tinned copper wire braided PVC outer sheath shipboard symmetrical communication cable ,Type SA |  |
| CHVV92/SA | 聚氯乙烯绝缘聚氯乙烯内护套镀锌钢丝编织铠装聚氯乙烯外护套船用对称式通信电缆，SA型 PVC insulation PVC inner sheath galvanized steel wire braided PVC outer sheath shipboard symmetrical communication cable ,Type SA |  |
| CHVVP/SA | 聚氯乙烯绝缘镀锡铜丝编织独立屏蔽聚氯乙烯护套船用对称式通信电缆，SA型 PVC insulation tinned copper wire braided individual screen PVC sheath shipboard symmetrical communication cable ,Type SA |  |
| CHVVP80/SA | 聚氯乙烯绝缘镀锡铜丝编织独立屏蔽聚氯乙烯内护套镀锡铜丝编织铠装船用对称式通信电缆，SA型 PVC insulation tinned copper wire braided individual screen PVC inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type SA |  |
| CHVVP90/SA | 聚氯乙烯绝缘镀锡铜丝编织独立屏蔽聚氯乙烯内护套镀锌钢丝编织铠装船用对称式通信电缆，SA型 PVC insulation tinned copper wire braided individual screen PVC inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type SA |  |
| CHVVP82/SA | 聚氯乙烯绝缘镀锡铜丝编织独立屏蔽聚氯乙烯内护套镀锡铜丝编织铠装聚氯乙烯外护套船用对称式通信电缆，SA型 PVC insulation tinned copper wire braided individual screen PVC inner sheath tinned copper wire braided PVC outer sheath shipboard symmetrical communication cable ,Type SA |  |
| CHVVP92/SA | 聚氯乙烯绝缘镀锡铜丝编织独立屏蔽聚氯乙烯内护套镀锌钢丝编织铠装聚氯乙烯外护套船用对称式通信电缆，SA型 PVC insulation tinned copper wire braided individual screen PVC inner sheath galvanized steel wire braided PVC outer sheath shipboard symmetrical communication cable ,Type SA |  |
| CHV8290/SA | 聚氯乙烯绝缘镀锡铜丝编织铠装聚氯乙烯内护套镀锌钢丝编织铠装船用对称式通信电缆，SA型 PVC insulation tinned copper wire braided PVC inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type SA |  |
| CHV8292/SA | 聚氯乙烯绝缘镀锡铜丝编织铠装聚氯乙烯内护套镀锌钢丝编织铠装聚氯乙烯外护套船用对称式通信电缆，SA型 PVC insulation tinned copper wire braided PVC inner sheath galvanized steel wire braided PVC outer sheath shipboard symmetrical communication cable ,Type SA |  |

| 型号 Type | 名称 Description | 产品标准 Standard |
| --- | --- | --- |
| CHEPJ/SC | 乙丙绝缘交联聚烯烃护套船用对称式通信电缆，SC型 EPR insulation cross-linked PO sheath shipboard symmetrical communication cable ,Type SC | IEC60092-350 IEC60092-376 IEC60332-3A |
| CHEPJ80/SC | 乙丙绝缘交联聚烯烃内护套镀锡铜丝编织铠装船用对称式通信电缆，SC型 EPR insulation cross-linked PO inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type SC |  |
| CHEPJ90/SC | 乙丙绝缘交联聚烯烃内护套镀锌钢丝编织铠装船用对称式通信电缆，SC型 EPR insulation cross-linked PO inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type SC |  |
| CHEPJ85/SC | 乙丙绝缘交联聚烯烃内护套镀锡铜丝编织铠装交联聚烯烃外护套船用对称式通信电缆，SC型 EPR insulation cross-linked PO inner sheath tinned copper wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type SC |  |
| CHEPJ95/SC | 乙丙绝缘交联聚烯烃内护套镀锌钢丝编织铠装交联聚烯烃外护套船用对称式通信电缆，SC型 EPR insulation cross-linked PO inner sheath galvanized steel wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type SC |  |
| CHEPJP/SC | 乙丙绝缘镀锡铜丝编织独立屏蔽交联聚烯烃护套船用对称式通信电缆，SC型 EPR insulation tinned copper wire braided individual screen cross-linked PO sheath shipboard symmetrical communication cable ,Type SC |  |
| CHEPJP80/SC | 乙丙绝缘镀锡铜丝编织独立屏蔽交联聚烯烃内护套镀锡铜丝编织铠装船用对称式通信电缆，SC型 EPR insulation tinned copper wire braided individual screen cross-linked PO inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type SC |  |
| CHEPJP90/SC | 乙丙绝缘镀锡铜丝编织独立屏蔽交联聚烯烃内护套镀锌钢丝编织铠装船用对称式通信电缆，SC型 EPR insulation tinned copper wire braided individual screen cross-linked PO inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type SC |  |
| CHEPJP85/SC | 乙丙绝缘镀锡铜丝编织独立屏蔽交联聚烯烃内护套镀锡铜丝编织铠装交联聚烯烃外护套船用对称式通信电缆，SC型 EPR insulation tinned copper wire braided individual screen cross-linked PO inner sheath tinned copper wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type SC |  |
| CHEPJP95/SC | 乙丙绝缘镀锡铜丝编织独立屏蔽交联聚烯烃内护套镀锌钢丝编织铠装交联聚烯烃外护套船用对称式通信电缆，SC型 EPR insulation tinned copper wire braided individual screen cross-linked PO inner sheath galvanized steel wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type SC |  |

| 型号 Type | 名称 Description | 产品标准 Standard |
| --- | --- | --- |
| CHEPF/SC | 乙丙绝缘聚烯烃护套船用对称式通信电缆，SC型 EPR insulation PO sheath shipboard symmetrical communication cable ,Type SC | IEC60092-350 IEC60092-376 IEC60332-3A |
| CHEPF80/SC | 乙丙绝缘聚烯烃内护套镀锡铜丝编织铠装船用对称式通信电缆，SC型 EPR insulation PO inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type SC |  |
| CHEPF90/SC | 乙丙绝缘聚烯烃内护套镀锌钢丝编织铠装船用对称式通信电缆，SC型 EPR insulation PO inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type SC |  |
| CHEPF86/SC | 乙丙绝缘聚烯烃内护套镀锡铜丝编织铠装交联聚烯烃外护套船用对称式通信电缆，SC型 EPR insulation PO inner sheath tinned copper wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type SC |  |
| CHEPF96/SC | 乙丙绝缘聚烯烃内护套镀锌钢丝编织铠装交联聚烯烃外护套船用对称式通信电缆，SC型 EPR insulation PO inner sheath galvanized steel wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type SC |  |
| CHEPFP/SC | 乙丙绝缘镀锡铜丝编织独立屏蔽聚烯烃护套船用对称式通信电缆，SC型 EPR insulation tinned copper wire braided individual screen PO sheath shipboard symmetrical communication cable ,Type SC |  |
| CHEPFP80/SC | 乙丙绝缘镀锡铜丝编织独立屏蔽聚烯烃内护套镀锡铜丝编织铠装船用对称式通信电缆，SC型 EPR insulation tinned copper wire braided individual screen PO inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type SC |  |
| CHEPFP90/SC | 乙丙绝缘镀锡铜丝编织独立屏蔽聚烯烃内护套镀锌钢丝编织铠装船用对称式通信电缆，SC型 EPR insulation tinned copper wire braided individual screen PO inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type SC |  |
| CHEPFP86/SC | 乙丙绝缘镀锡铜丝编织独立屏蔽聚烯烃内护套镀锡铜丝编织铠装聚烯烃外护套船用对称式通信电缆，SC型 EPR insulation tinned copper wire braided individual screen PO inner sheath tinned copper wire braided PO outer sheath shipboard symmetrical communication cable ,Type SC |  |
| CHEPFP96/SC | 乙丙绝缘镀锡铜丝编织独立屏蔽聚烯烃内护套镀锌钢丝编织铠装聚烯烃外护套船用对称式通信电缆，SC型 EPR insulation tinned copper wire braided individual screen PO inner sheath galvanized steel wire braided PO outer sheath shipboard symmetrical communication cable ,Type SC |  |

| 型号 Type | 名称 Description | 产品标准 Standard |
| --- | --- | --- |
| CHJV/SA | 交联聚乙烯绝缘聚氯乙烯护套船用对称式通信电缆，SA型 XLPE insulation PVC sheath shipboard symmetrical communication cable ,Type SA | IEC60092-350 IEC60092-376 IEC60332-3A |
| CHJV80/SA | 交联聚乙烯绝缘聚氯乙烯内护套镀锡铜丝编织铠装船用对称式通信电缆，SA型 XLPE insulation PVC inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type SA |  |
| CHJV90/SA | 交联聚乙烯绝缘聚氯乙烯内护套镀锌钢丝编织铠装船用对称式通信电缆，SA型 XLPE insulation PVC inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type SA |  |
| CHJV82/SA | 交联聚乙烯绝缘聚氯乙烯内护套镀锡铜丝编织铠装聚氯乙烯外护套船用对称式通信电缆，SA型 XLPE insulation PVC inner sheath tinned copper wire braided PVC outer sheath shipboard symmetrical communication cable ,Type SA |  |
| CHJV92/SA | 交联聚乙烯绝缘聚氯乙烯内护套镀锌钢丝编织铠装聚氯乙烯外护套船用对称式通信电缆，SA型 XLPE insulation PVC inner sheath galvanized steel wire braided PVC outer sheath shipboard symmetrical communication cable ,Type SA |  |
| CHJVP/SA | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚氯乙烯护套船用对称式通信电缆，SA型 XLPE insulation tinned copper wire braided individual screen PVC sheath shipboard symmetrical communication cable ,Type SA |  |
| CHJVP80/SA | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚氯乙烯内护套镀锡铜丝编织铠装船用对称式通信电缆，SA型 XLPE insulation tinned copper wire braided individual screen PVC inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type SA |  |
| CHJVP90/SA | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚氯乙烯内护套镀锌钢丝编织铠装船用对称式通信电缆，SA型 XLPE insulation tinned copper wire braided individual screen PVC inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type SA |  |
| CHJVP82/SA | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚氯乙烯内护套镀锡铜丝编织铠装聚氯乙烯外护套船用对称式通信电缆，SA型 XLPE insulation tinned copper wire braided individual screen PVC inner sheath tinned copper wire braided PVC outer sheath shipboard symmetrical communication cable ,Type SA |  |
| CHJVP92/SA | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚氯乙烯内护套镀锌钢丝编织铠装聚氯乙烯外护套船用对称式通信电缆，SA型 XLPE insulation tinned copper wire braided individual screen PVC inner sheath galvanized steel wire braided PVC outer sheath shipboard symmetrical communication cable ,Type SA |  |

| 型号 Type | 名称 Description | 产品标准 Standard |
| --- | --- | --- |
| CHJPJ/SC | 交联聚乙烯绝缘交联聚烯烃护套船用对称式通信电缆，SC型 XLPE insulation cross-linked PO sheath shipboard symmetrical communication cable ,Type SC | IEC60092-350 IEC60092-376 IEC60332-3A |
| CHJPJ80/SC | 交联聚乙烯绝缘交联聚烯烃内护套镀锡铜丝编织铠装船用对称式通信电缆，SC型 XLPE insulation cross-linked PO inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type SC |  |
| CHJPJ90/SC | 交联聚乙烯绝缘交联聚烯烃内护套镀锌钢丝编织铠装船用对称式通信电缆，SC型 XLPE insulation cross-linked PO inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type SC |  |
| CHJPJ85/SC | 交联聚乙烯绝缘交联聚烯烃内护套镀锡铜丝编织铠装交联聚烯烃外护套船用对称式通信电缆，SC型 EPR insulation cross-linked PO inner sheath tinned copper wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type SC |  |
| CHJPJ95/SC | 交联聚乙烯绝缘交联聚烯烃内护套镀锌钢丝编织铠装交联聚烯烃外护套船用对称式通信电缆，SC型 XLPE insulation cross-linked PO inner sheath galvanized steel wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type SC |  |
| CHJPJP/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽交联聚烯烃护套船用对称式通信电缆，SC型 EPR insulation tinned copper wire braided individual screen cross-linked PO sheath shipboard symmetrical communication cable ,Type SC |  |
| CHJPJP80/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽交联聚烯烃内护套镀锡铜丝编织铠装船用对称式通信电缆，SC型 XLPE insulation tinned copper wire braided individual screen cross-linked PO inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type SC |  |
| CHJPJP90/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽交联聚烯烃内护套镀锌钢丝编织铠装船用对称式通信电缆，SC型 XLPE insulation tinned copper wire braided individual screen cross-linked PO inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type SC |  |
| CHJPJP85/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽交联聚烯烃内护套镀锡铜丝编织铠装交联聚烯烃外护套船用对称式通信电缆，SC型 XLPE insulation tinned copper wire braided individual screen cross-linked PO inner sheath tinned copper wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type SC |  |
| CHJPJP95/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽交联聚烯烃内护套镀锌钢丝编织铠装交联聚烯烃外护套船用对称式通信电缆，SC型 XLPE insulation tinned copper wire braided individual screen cross-linked PO inner sheath galvanized steel wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type SC |  |
| CHJ85/SC | 交联聚乙烯绝缘镀锡铜丝编织铠装交联聚烯烃外护套船用对称式通信电缆，SC型 EPR insulation tinned copper wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type SC |  |
| CHJP85/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽镀锡铜丝编织铠装交联聚烯烃外护套船用对称式通信电缆，SC型 XLPE insulation tinned copper wire braided individual screen tinned copper wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type SC |  |

| 型号 Type | 名称 Description | 产品标准 Standard |
| --- | --- | --- |
| CHJPF/SC | 交联聚乙烯绝缘聚烯烃护套船用对称式通信电缆，SC型 XLPE insulation PO sheath shipboard symmetrical communication cable ,Type SC | IEC60092-350 IEC60092-376 IEC60332-3A |
| CHJPF80/SC | 交联聚乙烯绝缘聚烯烃内护套镀锡铜丝编织铠装船用对称式通信电缆，SC型 XLPE insulation PO inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type SC |  |
| CHJPF90/SC | 交联聚乙烯绝缘聚烯烃内护套镀锌钢丝编织铠装船用对称式通信电缆，SC型 XLPE insulation PO inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type SC |  |
| CHJPF86/SC | 交联聚乙烯绝缘聚烯烃内护套镀锡铜丝编织铠装聚烯烃外护套船用对称式通信电缆，SC型 XLPE insulation PO inner sheath tinned copper wire braided PO outer sheath shipboard symmetrical communication cable ,Type SC |  |
| CHJPF96/SC | 交联聚乙烯绝缘聚烯烃内护套镀锌钢丝编织铠装聚烯烃外护套船用对称式通信电缆，SC型 XLPE insulation PO inner sheath galvanized steel wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type SC |  |
| CHJPFP/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚烯烃护套船用对称式通信电缆，SC型 XLPE insulation tinned copper wire braided individual screen PO sheath shipboard symmetrical communication cable ,Type SC |  |
| CHJPFP80/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚烯烃内护套镀锡铜丝编织铠装船用对称式通信电缆，SC型 XLPE insulation tinned copper wire braided individual screen PO inner sheath tinned copper wire braided shipboard symmetrical communication cable ,Type SC |  |
| CHJPFP90/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚烯烃内护套镀锌钢丝编织铠装船用对称式通信电缆，SC型 XLPE insulation tinned copper wire braided individual screen PO inner sheath galvanized steel wire braided shipboard symmetrical communication cable ,Type SC |  |
| CHJPFP86/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚烯烃内护套镀锡铜丝编织铠装聚烯烃外护套船用对称式通信电缆，SC型 XLPE insulation tinned copper wire braided individual screen PO inner sheath tinned copper wire braided PO outer sheath shipboard symmetrical communication cable ,Type SC |  |
| CHJPFP96/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚烯烃内护套镀锌钢丝编织铠装聚烯烃外护套船用对称式通信电缆，SC型 XLPE insulation tinned copper wire braided individual screen PO inner sheath galvanized steel wire braided PO outer sheath shipboard symmetrical communication cable ,Type SC |  |
| CHJ86/SC | 交联聚乙烯绝缘镀锡铜丝编织铠装聚烯烃外护套船用对称式通信电缆，SC型 XLPE insulation PO inner sheath tinned copper wire braided PO outer sheath shipboard symmetrical communication cable ,Type SC |  |
| CHJP86/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽镀锡铜丝编织铠装聚烯烃外护套船用对称式通信电缆，SC型 XLPE insulation tinned copper wire braided individual screen PO inner sheath tinned copper wire braided PO outer sheath shipboard symmetrical communication cable ,Type SC |  |

| 型号 Type | 名称 Description | 产品标准 Standard |
| --- | --- | --- |
| CHJP85/SC （铝塑复合箔独立屏蔽） | 交联聚乙烯绝缘铝塑复合箔加泄漏线独立屏蔽镀镀锡铜丝编织铠装交联聚烯烃外护套船用对称式通信电缆，SC型 XLPE insulation aluminium/polyester foil with tinned copper drain wire individual screen tinned copper wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type SC | IEC60092-350 IEC60092-376 IEC60332-3A |
| CHJP86/SC（铝塑复合箔独立屏蔽） | 交联聚乙烯绝缘铝塑复合箔加泄漏线独立屏蔽镀锡铜丝编织铠装聚烯烃外护套船用对称式通信电缆，SC型 XLPE insulation aluminium/polyester foil with tinned copper drain wire individual screen tinned copper wire braided PO outer sheath shipboard symmetrical communication cable ,Type SC |  |
| CHJPJP85/SC（铝塑复合箔独立屏蔽） | 交联聚乙烯绝缘铝塑复合箔加泄漏线独立屏蔽交联聚烯烃内护套镀锡铜丝编织铠装交联聚烯烃外护套船用对称式通信电缆，SC型 XLPE insulation aluminium/polyester foil with tinned copper drain wire individual screen cross-linked PO inner sheath tinned copper wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type SC |  |
| CHJPJP85/SC（铝塑复合箔整体屏蔽） | 交联聚乙烯绝缘铝塑复合箔加泄漏线整体屏蔽交联聚烯烃内护套镀锡铜丝编织铠装交联聚烯烃外护套船用对称式通信电缆，SC型 XLPE insulation aluminium/polyester foil with tinned copper drain wire collectively screen cross-linked PO inner sheath tinned copper wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type SC |  |
| CHJPJP85/SC（铝塑复合箔独立+整体屏蔽） | 交联聚乙烯绝缘铝塑复合箔加泄漏线独立屏蔽整体屏蔽交联聚烯烃内护套镀锡铜丝编织铠装交联聚烯烃外护套船用对称式通信电缆，SC型 XLPE insulation aluminium/polyester foil with tinned copper drain wire individual and collectively screen cross-linked PO inner sheath tinned copper wire braided cross-linked PO outer sheath shipboard symmetrical communication cable ,Type SC |  |
| CHJPFP86/SC（铝塑复合箔独立屏蔽） | 交联聚乙烯绝缘铝塑复合箔加泄漏线独立屏蔽聚烯烃内护套镀锡铜丝编织铠装聚烯烃外护套船用对称式通信电缆，SC型 XLPE insulation aluminium/polyester foil with tinned copper drain wire individual screen PO inner sheath tinned copper wire braided PO outer sheath shipboard symmetrical communication cable ,Type SC |  |
| CHJPFP86/SC（铝塑复合箔整体屏蔽） | 交联聚乙烯绝缘铝塑复合箔加泄漏线整体屏蔽聚烯烃内护套镀锡铜丝编织铠装聚烯烃外护套船用对称式通信电缆，SC型 XLPE insulation aluminium/polyester foil with tinned copper drain wire collectively screen PO inner sheath tinned copper wire braided PO outer sheath shipboard symmetrical communication cable ,Type SC |  |
| CHJPFP86/SC（铝塑复合箔独立+整体屏蔽） | 交联聚乙烯绝缘铝塑复合箔加泄漏线独立屏蔽整体屏蔽聚烯烃内护套镀锡铜丝编织铠装聚烯烃外护套船用对称式通信电缆，SC型 XLPE insulation aluminium/polyester foil with tinned copper drain wire individual and collectively screen PO inner sheath tinned copper wire braided PO outer sheath shipboard symmetrical communication cable ,Type SC |  |