Standards：

IEC60092-350

IEC60092-351

IEC60092-352

IEC60092-353

IEC60092-359

IEC60228

IEC60332

Application：

The cable is intended for power, lighting and control system of   shipboard and off-shore building, and it is also available to metallurgical   industry, chemical works, power plant and mine etc.

1-conductorTinned copper wire

2-InsulationXLPE

3-Filler

4-Tape

5-Inner sheathPO/cross-linked PO

6-ArmourTinned copper wire or galvanized

7-Outer sheathPO/cross-linked PO

| 型号 Type | 名称 Description | 产品标准 Standard |
| --- | --- | --- |
| CEPF/SC | 乙丙绝缘聚烯径护套船用电力电缆，SC型 EPR insulation PO sheath shipboard power cable, Type SC | IEC60092-350 IEC60092-353 IEC60332-3A |
| CEPF80/SC | 乙丙绝缘聚烯径内护套镀锡铜丝编织铠装船用电力电缆，SC型 EPR insulation PO sheath tinned copper wire braided shipboard power cable, Type SC |  |
| CEPF90/SC | 乙丙绝缘聚烯径内护套镀锌钢丝编织铠装船用电力电缆，SC型 EPR insulation PO sheath galvanized steel wire braided shipboard power cable, Type SC |  |
| CEPF86/SC | 乙丙绝缘聚烯径内护套镀锡铜丝编织铠装聚烯径外护套船用电力电缆，SC型 EPR insulation PO inner sheath tinned copper wire braided PO outer sheath shipboard power cable, Type SC |  |
| CEPF96/SC | 乙丙绝缘聚烯径内护套镀锌钢丝编织铠装聚烯径外护套船用电力电缆，SC型 EPR insulation PO inner sheath galvanized steel wire braided PO outer sheath shipboard power cable, Type SC |  |
| CEPJ/SC | 乙丙绝缘交联聚烯径护套船用电力电缆，SC型 EPR insulation cross-linked PO sheath shipboard power cable, Type SC |  |
| CEPJ80/SC | 乙丙绝缘交联聚烯径内护套镀锡铜丝编织铠装船用电力电缆，SC型 EPR insulated cross-linked PO sheath tinned copper wire braided shipboard power cable, Type SC |  |
| CEPJ90/SC | 乙丙绝缘交联聚烯径内护套镀锌钢丝编织铠装船用电力电缆，SC型 EPR insulated cross-linked PO sheath galvanized steel wire braided shipboard power cable, Type SC |  |
| CEPJ85/SC | 乙丙绝缘交联聚烯径内护套镀锡铜丝编织铠装交联聚烯径外护套船用电力电缆，SC型 EPR insulated cross-linked PO inner sheath tinned copper wire braided cross-linked PO outer sheath shipboard power cable, Type SC |  |
| CEPJ95/SC | 乙丙绝缘交联聚烯径内护套镀锌钢丝编织铠装交联聚烯径外护套船用电力电缆，SC型 EPR insulation cross-linked PO inner sheath galvanized steel wire braided cross-linked PO outer sheath shipboard power cable, Type SC |  |
| CEPJR/SC | 乙丙绝缘交联聚烯径护套船用电力软电缆，SC型 EPR insulation cross-linked PO sheath shipboard flexible power cable, Type SC |  |
| CEPFR/SC | 乙丙绝缘聚烯径护套船用电力软电缆，SC型 EPR insulation PO sheath shipboard flexible power cable, Type SC |  |

| 型号 Type | 名称 Description | 产品标准 Standard |
| --- | --- | --- |
| CJPF/SC | 交联聚乙烯绝缘聚烯径护套船用电力电缆，SC型 XLPE insulation PO sheath shipboard power cable, Type SC | IEC60092-350 IEC60092-353 IEC60332-3A |
| CJPF80/SC | 交联聚乙烯绝缘聚烯径内套镀锡铜丝编织铠装船用电力电缆，SC型 XLPE insulation PO sheath tinned copper wire braided shipboard power cable, Type SC |  |
| CJPF90/SC | 交联聚乙烯绝缘聚烯径内套镀锌钢丝编织铠装船用电力电缆，SC型 XLPE insulation PO sheath galvanized steel wire braided shipboard power cable, Type SC |  |
| CJPF86/SC | 交联聚乙烯绝缘聚烯径内套镀锡铜丝编织铠装聚烯径外套船用电力电缆，SC型 XLPE insulation PO inner sheath tinned copper wire braided PO outer sheathed shipboard power cable, Type SC |  |
| CJPF96/SC | 交联聚乙烯绝缘聚烯径内套镀锌钢丝编织铠装聚烯径外套船用电力电缆，SC型 XLPE insulation PO inner sheath galvanized steel wire braided PO outer sheathed shipboard power cable, Type SC |  |
| CJPJ/SC | 交联聚乙烯绝缘交联聚烯径护套船用电力电缆，SC型 XLPE insulation cross-linked PO sheath shipboard power cable, Type SC |  |
| CJPJ80/SC | 交联聚乙烯绝缘交联聚烯径内套镀锡铜丝编织铠装船用电力电缆，SC型 XLPE insulated cross-linked PO sheathed tinned copper wire braided shipboard power cable, Type SC |  |
| CJPJ90/SC | 交联聚乙烯绝缘交联聚烯径内套镀锌钢丝编织铠装船用电力电缆，SC型 XLPE insulated cross-linked PO sheathed galvanized steel wire braided shipboard power cable, Type SC |  |
| CJPJ85/SC | 交联聚乙烯绝缘交联聚烯径内套镀锡铜丝编织铠装交联聚烯径外套船用电力电缆，SC型 XLPE insulated cross-linked PO inner sheathed tinned copper wire braided cross-linked PO outer sheathed shipboard power cable, Type SC |  |
| CJPJ95/SC | 交联聚乙烯绝缘交联聚烯径内套镀锌钢丝编织铠装交联聚烯径外套船用电力电缆，SC型 XLPE insulation cross-linked PO inner sheath galvanized steel wire braided cross-linked PO outer sheath shipboard power cable, Type SC |  |
| CJPJR/SC | 交联聚乙烯绝缘交联聚烯径护套船用电力软电缆，SC型 XLPE insulation cross-linked PO sheath shipboard flexible power cable, Type SC |  |
| CJPFR/SC | 交联聚乙烯绝缘聚烯径护套船用电力软电缆，SC型 XLPE insulation PO sheath shipboard flexible power cable, Type SC |  |

| 型号 Type | 名称 Description | 产品标准 Standard |
| --- | --- | --- |
| CEPFP/SC | 乙丙绝缘镀锡铜丝编织独立屏蔽聚烯径护套船用电力电缆，SC型 EPR insulation tinned copper wire braided individual screen PO sheath shipboard power cable, Type SC | IEC60092-350 IEC60092-353 IEC60332-3A |
| CEPFP80/SC | 乙丙绝缘镀锡铜丝编织独立屏蔽聚烯径内套镀锡铜丝编织铠装船用电力电缆，SC型 EPR insulation tinned copper wire braided individual screen PO sheath tinned copper wire braided shipboard power cable, Type SC |  |
| CEPFP90/SC | 乙丙绝缘镀锡铜丝编织独立屏蔽聚烯径内套镀锌钢丝编织铠装船用电力电缆，SC型 EPR insulation tinned copper wire braided individual screen PO sheath galvanized steel wire braided shipboard power cable, Type SC |  |
| CEPFP86/SC | 乙丙绝缘镀锡铜丝编织独立屏蔽聚烯径内套镀锡铜丝编织铠装聚烯径外套船用电力电缆，SC型 EPR insulation tinned copper wire braided individual screen PO inner sheath tinned copper wire braided PO outer sheath shipboard power cable, Type SC |  |
| CEPFP96/SC | 乙丙绝缘镀锡铜丝编织独立屏蔽聚烯径内套镀锌钢丝编织铠装聚烯径外套船用电力电缆，SC型 EPR insulation tinned copper wire braided individual screen PO inner sheath galvanized steel wire braided PO outer sheath shipboard power cable, Type SC |  |
| CEPJP/SC | 乙丙绝缘镀锡铜丝编织独立屏蔽交联聚烯径护套船用电力电缆，SC型 EPR insulation tinned copper wire braided individual screen cross-linked PO sheath shipboard power cable, Type SC |  |
| CEPJP80/SC | 乙丙绝缘镀锡铜丝编织独立屏蔽交联聚烯径内套镀锡铜丝编织铠装船用电力电缆，SC型 EPR insulated tinned copper wire braided individual screen cross-linked PO sheath tinned copper wire braided shipboard power cable, Type SC |  |
| CEPJP90/SC | 乙丙绝缘镀锡铜丝编织独立屏蔽交联聚烯径内套镀锌钢丝编织铠装船用电力电缆，SC型 EPR insulated tinned copper wire braided individual screen cross-linked PO sheath galvanized steel wire braided shipboard power cable, Type SC |  |
| CEPJP85/SC | 乙丙绝缘镀锡铜丝编织独立屏蔽交联聚烯径内套镀锡铜丝编织铠装交联聚烯径外套船用电力电缆，SC型 EPR insulated tinned copper wire braided individual screen cross-linked PO inner sheath tinned copper wire braided cross-linked PO outer sheath shipboard power cable, Type SC |  |
| CEPJP95/SC | 乙丙绝缘镀锡铜丝编织独立屏蔽交联聚烯径内套镀锌钢丝编织铠装交联聚烯径外套船用电力电缆，SC型 EPR insulation tinned copper wire braided individual screen cross-linked PO inner sheath galvanized steel wire braided cross-linked PO outer sheath shipboard power cable, Type SC |  |

| 型号 Type | 名称 Description | 产品标准 Standard |
| --- | --- | --- |
| CJPFP/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚烯径护套船用电力电缆，SC型 XLPE insulation tinned copper wire braided individual screen PO sheath shipboard power cable, Type SC | IEC60092-350 IEC60092-353 IEC60332-3A |
| CJPFP80/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚烯径内套镀锡铜丝编织铠装船用电力电缆，SC型 XLPE insulation tinned copper wire braided individual screen PO sheath tinned copper wire braided shipboard power cable, Type SC |  |
| CJPFP90/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚烯径内套镀锌钢丝编织铠装船用电力电缆，SC型 XLPE insulation tinned copper wire braided individual screen PO sheath galvanized steel wire braided shipboard power cable, Type SC |  |
| CJPFP86/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚烯径内套镀锡铜丝编织铠装聚烯径外套船用电力电缆，SC型 XLPE insulation tinned copper wire braided individual screen PO inner sheath tinned copper wire braided PO outer sheathed shipboard power cable, Type SC |  |
| CJPFP96/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚烯径内套镀锌钢丝编织铠装聚烯径外套船用电力电缆，SC型 XLPE insulation tinned copper wire braided individual screen PO inner sheath galvanized steel wire braided PO outer sheathed shipboard power cable, Type SC |  |
| CJPJP/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽交联聚烯径护套船用电力电缆，SC型 XLPE insulation tinned copper wire braided individual screen cross-linked PO sheath shipboard power cable, Type SC |  |
| CJPJP80/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽交联聚烯径内套镀锡铜丝编织铠装船用电力电缆，SC型 XLPE insulated tinned copper wire braided individual screen cross-linked PO sheathed tinned copper wire braided shipboard power cable, Type SC |  |
| CJPJP90/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽交联聚烯径内套镀锌钢丝编织铠装船用电力电缆，SC型 XLPE insulated tinned copper wire braided individual screen cross-linked PO sheathed galvanized steel wire braided shipboard power cable, Type SC |  |
| CJPJP85/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽交联聚烯径内套镀锡铜丝编织铠装交联聚烯径外套船用电力电缆，SC型 XLPE insulated tinned copper wire braided individual screen cross-linked PO inner sheathed tinned copper wire braided cross-linked PO outer sheathed shipboard power cable, Type SC |  |
| CJPJP95/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽交联聚烯径内套镀锌钢丝编织铠装交联聚烯径外套船用电力电缆，SC型 XLPE insulation tinned copper wire braided individual screen cross-linked PO inner sheath galvanized steel wire braided cross-linked PO outer sheath shipboard power cable, Type SC |  |

| 型号 Type | 名称 Description | 产品标准 Standard |
| --- | --- | --- |
| CJPFP/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚烯径护套船用电力电缆，SC型 XLPE insulation tinned copper wire braided individual screen PO sheath shipboard power cable, Type SC | IEC60092-350 IEC60092-353 IEC60332-3A |
| CJPFP80/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚烯径内套镀锡铜丝编织铠装船用电力电缆，SC型 XLPE insulation tinned copper wire braided individual screen PO sheath tinned copper wire braided shipboard power cable, Type SC |  |
| CJPFP90/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚烯径内套镀锌钢丝编织铠装船用电力电缆，SC型 XLPE insulation tinned copper wire braided individual screen PO sheath galvanized steel wire braided shipboard power cable, Type SC |  |
| CJPFP86/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚烯径内套镀锡铜丝编织铠装聚烯径外套船用电力电缆，SC型 XLPE insulation tinned copper wire braided individual screen PO inner sheath tinned copper wire braided PO outer sheathed shipboard power cable, Type SC |  |
| CJPFP96/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽聚烯径内套镀锌钢丝编织铠装聚烯径外套船用电力电缆，SC型 XLPE insulation tinned copper wire braided individual screen PO inner sheath galvanized steel wire braided PO outer sheathed shipboard power cable, Type SC |  |
| CJPJP/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽交联聚烯径护套船用电力电缆，SC型 XLPE insulation tinned copper wire braided individual screen cross-linked PO sheath shipboard power cable, Type SC |  |
| CJPJP80/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽交联聚烯径内套镀锡铜丝编织铠装船用电力电缆，SC型 XLPE insulated tinned copper wire braided individual screen cross-linked PO sheathed tinned copper wire braided shipboard power cable, Type SC |  |
| CJPJP90/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽交联聚烯径内套镀锌钢丝编织铠装船用电力电缆，SC型 XLPE insulated tinned copper wire braided individual screen cross-linked PO sheathed galvanized steel wire braided shipboard power cable, Type SC |  |
| CJPJP85/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽交联聚烯径内套镀锡铜丝编织铠装交联聚烯径外套船用电力电缆，SC型 XLPE insulated tinned copper wire braided individual screen cross-linked PO inner sheathed tinned copper wire braided cross-linked PO outer sheathed shipboard power cable, Type SC |  |
| CJPJP95/SC | 交联聚乙烯绝缘镀锡铜丝编织独立屏蔽交联聚烯径内套镀锌钢丝编织铠装交联聚烯径外套船用电力电缆，SC型 XLPE insulation tinned copper wire braided individual screen cross-linked PO inner sheath galvanized steel wire braided cross-linked PO outer sheath shipboard power cable, Type SC |  |